Technical specifications for a forklift for the Bektakari mine

CONTENTS

SECTION 1. GENERAL INFORMATION

Subsection 1.1 Name

Subsection 1.2 Information on novelty

SECTION 2. APPLICATION SCOPE

SECTION 3. OPERATING CONDITIONS

SECTION 4. TECHNICAL REQUIREMENTS

Subsection 4.1. Main technical, economic and operational indicators

Subsection 4.2. Reliability requirements

Subsection 4.3. Requirements for materials and components of the vehicle

Subsection 4.4. Power supply requirements

Subsection 4.5. Documentation requirements

SECTION 5. REQUIREMENTS TO THE SCOPE AND/OR PERIOD OF WARRANTY

SECTION 6. REQUIREMENTS TO REPAIRABILITY

SECTION 7. ENVIRONMENTAL REQUIREMENTS

SECTION 8. SAFETY REQUIREMENTS

1. GENERAL INFORMATION

1.1 Name

Universal self-propelled special equipment, a type of forklift, designed for loading and transporting various materials.

1.2 Information about the novelty

The supplier shall ensure that the equipment supplied is new, previously unused, a production model reflecting all the latest modifications and not discontinued by the manufacturer at the time of delivery.

2. APPLICATION SCOPE

For transportation of unloading materials.

3. OPERATING CONDITIONS

For all macroclimatic regions on land, except for the climate region with Antarctic cold climate

4. TECHNICAL REQUIREMENTS

4.1 Main parameters and dimensions

| Standard lifting height | 3000 mm. |
| --- | --- |
| Maximum lifting height | 6500 mm. |
| Loading center | 500 mm. |
| Free movement of forks | 165 mm |
| Forks (length, width, thickness) | 1200 х 125 х 45 mm. |
| Fork adjustment | 200 х 1300 mm. |
| Mast tilt (forward, backward) gr. | 6-12 |
| Front overhang | 600 mm. |
| Rear overhang | 600 mm. |
| Minimum ground clearance under the mast | 220mm. |
| Length to the back of the forks | 1220 mm. |
| Overall width | 1650 mm |
| Mast height in the folded position | 2100 mm. |
| Mast height in the raised position | 6000 mm. |
| Load capacity | 3500 kg |
| Engine |  |
| Engine type | Liquid-cooled diesel |
| Drive type |  |
| Full-wheel drive 4x4 with connected bridge and lowering gear |  |
| Transmission |  |
| Hydrostatic |  |
| Rear track | 1720 mm. |
| Front track | 1720 mm |
| Tires |  |
| Front wheels | 12-16.5 |
| Rear wheels | 12-16.5 |
| Wheel type | Pneumatic |
| Steering wheel |  |
| Outer turning radius | 4500 mm. |
| Hydraulics |  |
| Dual-circuit hydraulic for additional attachments. |  |
| Electrical system |  |
| Operating voltage | 12 V |
| Battery | 12/90 A |
| Protection | 65 IP |
| Main and rear lights: | LED |
| Additional lights and signal lamps: | LED |
| Audible and visual alarm for reverse gear | Audible and visual alarm for reverse gear |
| Electrical wiring | Wiring harnesses in fireproof and waterproof casing |
| Operator cabin |  |
| External protection of the cabin from falling objects |  |
| Operator seat | Ergonomic seat with seat belt |
| Other equipment |  |
| Automatic fire extinguishing system |  |
| Fire extinguisher | Portable fire extinguisher 1 x 8 kg |
| Information signs | Warning signs and inscriptions in Russian and English |
| Signal lights | Flashing beacon |
| 4.1 Technical, economic and operational indicators |  |
| On-demand operation mode |  |
| 4.2 Reliability requirements |  |
| The standard service life is at least 4 years. |  |
| 4.3 Documentation requirements |  |
| Operation, maintenance and repair instructions on electronic and paper media. Operation, maintenance instructions for the internal combustion engine on electronic and paper media. Forklift passport. |  |
| 5. REQUIREMENTS TO THE SCOPE AND PERIOD OF WARRANTY |  |
| The warranty period for service is at least 12 months. No limitation on engine hours. |  |
| 6. REQUIREMENTS TO REPAIRABILITY |  |
| The loader parts and their connections must withstand the forces and stresses to which they are subjected during active operation. The durability of the materials used must correspond to the intended operation, taking into account the occurrence of danger associated with the phenomena of fatigue, aging, corrosion and wear. The operating manual (instructions) for the machine must specify the type and frequency of inspection and maintenance required to ensure safety. The parts, units, and assemblies subject to wear and tear, and the criteria for their assessment and replacement must be specified. |  |
| 7. ENVIRONMENTAL REQUIREMENTS |  |
| The exhaust gas content must comply with the Euro 3 standard |  |
| 8. SAFETY REQUIREMENTS |  |
| The forklift must be supplied in strict accordance with the "Safety Rules for Loading and Unloading Operations" |  |