Technical Specification for the Procurement of Equipment for

JSC «RMG GROUP»

| AGREED: Head of Madneuli factory JSC “RMG Copper”_ __________ K. Kekelidze «___» __________2025 г. AGREED: Chief Mechanical Engineer JSC «RMG Group» ___________ Giorgi Dzhakeli «___» __________2025 г. | APPROVED BY Production Director JSC «RMG Group» ___________ S. E. Berezikov «___» __________2025 г. |
| --- | --- |

Calculation, Supply and Installation of a 5-ton Electric Hoist

Tbilisi 2025

- Electric Hoist with 5-ton Capacity

  - Purpose

# Modernization of the lifting mechanism installed on Conveyor No. 3 in the bunker building of the concentrator plant.

# Elimination of the risk of hoist downtime during maintenance and repair works on Conveyor No. 3

  - Supplier Requirements

The supplier shall be a developer and manufacturer of lifting equipment.

The supplier shall have proven experience in the design, supply, manufacturing, and installation of lifting equipment, including equipment used at mining and mineral processing facilities.

  - Technical Description and Scope of Application

The lifting equipment is intended for performing lifting operations under workshop/industrial indoor conditions.

- INITIAL TECHNICAL DATA

| Electric Hoist – Lifting / Lowering |  |
| --- | --- |
| Rated load (Q) | 5 tons |
| Lifting height (Н) | 32 m |
| Standard (FЕМ,ISO) | 2 m/M5 |
| Operating conditions | Industrial workshop |
| Ambient temperature range | -10 +35 |
| Design | Monorail, suspended type |
| Hoist travel length, m | 120 |
| Max. lifting speed (m/min) | 8,00 -two speeds -frequency inverter |
| Min. lifting speed (m/min) | 2,00 |
| Dual-speed lifting | Two speeds -frequency inverter |
| Motor power / type (kW) | 7,5 kW (1шт.) |
| Motor speed (rpm) | 1500 - lP55 / F insulation,/ Squirrel-cage induction motor /4О% ED |
| Rope diameter (mm) | 12 - 6*З6 WARRINGTON Seale (200kg/mm²) |
| Reeving ratio | 4/2 |
| Rope type | Steel wire rope |
| Brake | Electromagnetic |
| Gearbox/Gear reducer | Reducer with Helical cylindrical gear (with helical teeth) |
| Hook | Hook with safety latch- (DlN 15401) |
| Electric Hoist – Travel |  |
| Max. travel speed (m/min) | 16,00 - two speeds -frequency inverter |
| Min. travel speed (m/min) | 4,00 |
| Dual-speed travel | Two speeds -frequency inverter |
| Motor power (kW) | 0,37 – 2 pcs |
| Motor speed (rpm) | 1500 - lP55 / F insulation,/ Squirrel-cage induction motor /4О% ED |
| Travel brake | Electromagnetic |
| Travel gearbox | Reducer with Helical cylindrical gear (with helical teeth) |
| Wheel diameter (mm) | 125 mm |
| Number of wheels | 4 |
| Driven wheels (pcs) | 2 |
| Wheel material | AISI 1040 |
| Control Panel |  |
| Supply voltage and frequency | 380-400 volt (±%5) 50/60 Hz |
| Control voltage | 24VAC |
| Control type | Radio remote control |
| Protection class | IP65 |
| Electric Hoist – Power Supply |  |
| Power supply to hoist | Busbar system (busduct) |
| Hoist bridge travel length, m | 120 |
| Number of hoists per beam. | 1 |
| Safety System |  |
| Overload limiter | Yes, load cell / indicator |
| Upper limit switch (lifting) | Yes |
| Travel limit switch | Yes |
| Anti-collision device | No |
| Thermal and phase protection | Yes |
| Emergency stop | Yes |
| Warning / signaling system | Yes |
| Buffers | Yes |
| Rope guide | Yes |

- Overall Dimensions

The overall dimensions are determined by calculation, necessary design solutions, the type of selected materials and components, and the design documentation approved by the Customer.

- Warranty Conditions

The warranty period is 12 months from the date of commissioning.

- Scope of Supply:

| № | Description of Main Items | Pcs |
| --- | --- | --- |
| 1 | Electric hoist | 1 |
| 2 | Trolley busbar (120 m) complete with necessary suspension and connection materials, hoist bridge power supply | 1 |
| 3 | Remote control pendant | 2 |
| 4 | Load limiter | 1 |

- List of Required Documentation

The supplier shall provide the Customer with the following technical documentation:

| № | Documentation | Delivery Time** |
| --- | --- | --- |
| 1 | User manual | With the equipment |
| 2 | Spare parts catalog | With the equipment |
| 3 | Electrical wiring diagrams and control panel schematics | With the equipment |
| 4 | Certificate of conformity and certificate of origin | With the equipment |

** Documentation must be provided electronically in DWG and PDF formats, in English, Russian languages.

- Packaging Requirements

Not regulated.

- Delivery Conditions

The goods shall be delivered under the following terms: DDP Kazreti station, Georgia.

| Prepared by: |  |  |
| --- | --- | --- |
| Chief Mechanic of the Copper Processing Plant JSC “RMG Group” |  | A. Tashcheev |
| Approved by: |  |  |
| Chief Engineer of the Copper Processing Plant JSC “RMG Group” |  | M. Beridze |
| Chief Power Engineer of the Copper Processing Plant JSC “RMG Group” |  | I. Zykov |
| Senior Mechanic of the Copper Processing Plant JSC “RMG Group” |  | Zh. Beridze |