Design, Supply and Supervision of Installation of the Drive Station for the Conveyor of Diemme F13 Filter Press Tbilisi, 2025 1. DRIVE STATION OF THE DIEMME F13 FILTER PRESS CONVEYOR 1.1 Purpose Modernization of the conveyor drive in order to ensure reliable operation of the Diemme F13 filter press. Elimination of risks of stoppage of the Diemme F13 filter press. Elimination of risks of conveyor stoppage under material blockage. Bringing the calculated operating parameters of the conveyor into compliance. 1.2 Requirements for the Supplier The supplier shall be a developer and manufacturer of conveyor drive stations. The supplier shall have experience in design, supply, manufacturing and installation of conveyor drive stations, including for mining and processing plants. The supplier shall send a specialist to the customer’s site to inspect the existing conveyor drive station, perform required measurements and clarifications, carry out calculations, and recommend the necessary engineering solutions and modifications. 1.3 Technical Description and Scope of Application The conveyor is intended for transportation of copper concentrate discharged from the Diemme F13 filter press. 2. INITIAL TECHNICAL DATA Parameter Value Conveyor operation 24 h/day Operating conditions Indoor workshop Ambient temperature range 0 to +35 °C Conveyed material Copper concentrate Belt speed, m/s 1.7 Conveyor capacity, t/h 1500 Belt width, mm 1200 Conveyor length, m 37 Lifting height, m Approx. 5 Material moisture % Bulk density, t/m³ 2.8 3. DIMENSIONS Overall dimensions shall be determined by calculation, required engineering solutions, type of selected materials and components, and design documentation agreed with the customer. 4. WARRANTY CONDITIONS Warranty period: 12 months from the date of commissioning. 5. SCOPE OF SUPPLY No. Description Qty 1 Drive pulley Ø500–1400 with rubber lagging, SKF/FAG bearings 1 2 Bend pulley Ø270–1400 with rubber lagging, SKF/FAG bearings 1 3 Gear motor Innomotics (Siemens) 45 kW (to be уточified by calculation), IP55, climatic temperature range -15 … +40 °C 2 4 Primary belt scraper (coarse cleaning) 1 5 Variable Frequency Drive Schneider Electric ATV950, IP55, including programming (pre-configuration and synchronization) 2 6 Steel structures: support frame for the drive pulley with enclosure and anticorrosion treatment 1 6. REQUIRED DOCUMENTATION The supplier shall provide the Customer with the following technical documentation: No. Documentation Delivery Time 1 Installation and operation manual With equipment 2 Spare parts catalogue With equipment 3 Assembly drawing of the drive section To be agreed with the customer before manufacturing 4 Electrical design (integration into the existing system, issuance of technical assignment for protective equipment installation, cable sizing, control circuit connection diagram) To be agreed with the customer before manufacturing 5 Certificate of conformity and origin With equipment Drawings shall be provided in electronic format (DWG and PDF). Documentation shall be provided in English and Russian.