| Technical Specifications for the Selection of an Auxiliary Fan for the Bektakari Mine |  |  |
| --- | --- | --- |
| 1 | Location of the auxiliary fan relative to sea level, meter | 600-700 above sea level |
| 2 | Fan installation site | Surface / Underground |
| 3 | Installation environment (Mine atmosphere) | Non-gassy |
| 4 | With a single electric motor | 90kw. |
| 5 | Temperature in the mine | 16° to 20°C |
| 6 | Surface temperature | minus 6°C to +35°C |
| 7 | Ingress protection rating of electrical equipment | IP55 |
| 8 | Design static pressure (calculated) | 2500 Pa |
| 9 | Required performance | Max. 37 m³/s |
| 10 | Ventilation method | Forcing |
| 11 | Outlet adapter diameter | 1400mm |
| 12 | Operating duty | Continuous (24/7) |
| 13 | Mains voltage | 400V |
| 14 | Frequency | 50Hz |
| 15 | Silencers on the suction and discharge sides | yes |
| 16 | Fan mounting: ground, suspended | yes |
| Management |  |  |
| 1 | By means of VFD (Variable Frequency Drive) | yes |
| 2 | Remote control / Local control | yes |
| 3 | VFD placement inside the cabinet | yes |
| Controlled parameter |  |  |
| 1 | Motor operating parameters | yes |
| 2 | Motor bearing lubrication points | yes |
| Mine information |  |  |
| 1 | Surface decline | yes |
| 2 | Mining method | drill and blast |
| 3 | All works using self-propelled internal combustion engine (ICE) equipment | yes |