# Tube To Tube Union

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 51.3 |
| Material | 316 Stainless Steel |
| Series | CUA |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Pipe Thread | ISO Tapered¦ISO Parallel¦NPT¦Unified Screw |
| Type | Connects Fractional Tubes |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 20.63 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 13/16 |
| Dimension d (mm) | 10.40 |

| Tube O.D. (in) | 1/8 |
| --- | --- |
| Tube O.D. (mm) | 3.17 |
| Dimension L (mm) | 35.6 |
| Material | 316 Stainless Steel |
| Series | CUA |
| Dimension A (mm) | 12.7 |
| Dimension B (mm) | 15.2 |
| Pipe Thread | ISO Tapered¦ISO Parallel¦NPT¦Unified Screw |
| Type | Connects Fractional Tubes |
| Width Across Flat (H) (mm) | 11.11 |
| Width Across Flat h (mm) | 11.11 |
| Width Across Flat (H) (in) | 7/16 |
| Width Across Flat h (in) | 7/16 |
| Dimension d (mm) | 2.30 |

# Tube To Tube Union Elbow

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 36.1 |
| Material | 316 Stainless Steel |
| Series | CLA |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Pipe Thread | ISO Tapered¦ISO Parallel¦NPT¦Unified Screw |
| Type | Connects Fractional Tubes |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 20.63 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 13/16 |
| Dimension d (mm) | 10.40 |

# Tube To Tube Union Tee

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 36.1 |
| Material | 316 Stainless Steel |
| Series | CTA |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Pipe Thread | ISO Tapered¦ISO Parallel¦NPT¦Unified Screw |
| Type | Connects Fractional Tubes |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 20.63 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 1/2 |
| Option | Reducing Union |
| Dimension d (mm) | 10.40 |

# Male Elbow Tube To Male

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 36.1 |
| Material | 316 Stainless Steel |
| Series | CLMA |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Dimension l (mm) | 25.9 |
| Min. d (mm) | 10.4 |
| Pipe Thread | ISO Tapered¦NPT |
| Type | Connects Fractional Tube to Female NPT Thread |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 20.63 |
| Quality System Certificates | ISO 9001 Certificate No. GQC 212, ASME Sect II (MO) Certificate No. QSC 584 |
| Dimension L₁ (mm) | 33.0 |
| T (NPT) (mm) | 1/2 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 13/16 |

# Male Connector Tube To Male

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 49.0 |
| Material | 316 Stainless Steel |
| Series | CMC - N |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Pipe Thread | NPT |
| Type | Connects Fractional Tube to Female NPT Thread |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 22.22 |
| T (NPT) (mm) | 1/2 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 7/8 |
| Option | Male Connector |
| Dimension d (mm) | 10.40 |

# 45 Deg Tube To Male Elbow

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 32.0 |
| Material | 316 Stainless Steel |
| Series | CLMB |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Dimension l (mm) | 21.8 |
| Min. d (mm) | 10.4 |
| Pipe Thread | ISO Tapered¦NPT |
| Type | Connects Fractional Tube to Female NPT Thread |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 20.63 |
| Quality System Certificates | ISO 9001 Certificate No. GQC 212, ASME Sect II (MO) Certificate No. QSC 584 |
| Dimension L₁ (mm) | 29.0 |
| T (NPT) (mm) | 1/2 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 13/16 |

# Male Run Tee Tube To Male

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 36.1 |
| Material | 316 Stainless Steel |
| Series | CRTM |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Dimension l (mm) | 25.9 |
| Min. d (mm) | 10.4 |
| Pipe Thread | ISO Tapered¦NPT |
| Type | Connects Fractional Tube to Female NPT Thread |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 20.63 |
| Quality System Certificates | ISO 9001 Certificate No. GQC 212, ASME Sect II (MO) Certificate No. QSC 584 |
| Dimension L₁ (mm) | 33.0 |
| T (NPT) (mm) | 1/2 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 13/16 |

# Male Branch Tee

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Dimension L (mm) | 36.1 |
| Material | 316 Stainless Steel |
| Series | CBTM |
| Dimension A (mm) | 22.9 |
| Dimension B (mm) | 21.8 |
| Dimension l (mm) | 25.9 |
| Min. d (mm) | 10.4 |
| Pipe Thread | ISO Tapered¦NPT |
| Type | Connects Fractional Tube to Female NPT Thread |
| Width Across Flat (H) (mm) | 22.22 |
| Width Across Flat h (mm) | 20.63 |
| Quality System Certificates | ISO 9001 Certificate No. GQC 212, ASME Sect II (MO) Certificate No. QSC 584 |
| Dimension L₁ (mm) | 33.0 |
| T (NPT) (mm) | 1/2 |
| Width Across Flat (H) (in) | 7/8 |
| Width Across Flat h (in) | 13/16 |

# Plug For Port

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Material | 316 Stainless Steel |
| Series | CPA |
| Pipe Thread | ISO Tapered¦ISO Parallel¦NPT¦Unified Screw |
| Type | Connects Fractional Hy-Lok Port |
| Width Across Flat (H) (mm) | 22.22 |
| Quality System Certificates | ISO 9001 Certificate No. GQC 212, ASME Sect II (MO) Certificate No. QSC 584 |
| Width Across Flat (H) (in) | 7/8 |
| Used For | Hy-Lok Port |

# Nut Ferrule Set

| Tube O.D. (in) | 1/2 |
| --- | --- |
| Tube O.D. (mm) | 12.7 |
| Material | 316 Stainless Steel |
| Series | CNFS |

| Tube O.D. (in) | 1/8 |
| --- | --- |
| Tube O.D. (mm) | 3.17 |
| Material | 316 Stainless Steel |
| Series | CNFS |