General Information:

Due to the planned upgrade of network devices to new and more advanced models, the procurement of network equipment is planned.

Equipment Requirements:

L2/L3 switches equivalent to Cisco Catalyst 9200L or higher, meeting or exceeding the specifications below.

- L2/L3 manageable switch

- Ports: 24 or 48 × 1G (RJ-45)

- Uplink ports: minimum 4 × SFP/SFP+ (1G/10G)

- Support for PoE/PoE+ (where applicable)

- PoE power budget: minimum 600 W (for PoE-capable models)

- Stacking capability (hardware-based stacking)

- L3 routing support (static, dynamic)

- Support for VLAN (802.1Q), STP/RSTP, QoS

- Support for AAA (RADIUS/TACACS+)

- Support for IPv4/IPv6 dual stack

- 24×7 manufacturer technical support for a minimum of 36 months.

Commercial and Compliance Requirements:

All equipment must be:

- Brand new

- Unused

- Not refurbished

- Not unauthorized import.

The bidder must provide a valid Manufacturer Authorization Letter (MAL) to confirm that equipment is officially

supplied by authorized channels and eligible for vendor support.

|  | Item name | Quantity | Delivery days |
| --- | --- | --- | --- |
| 1 | Switch 48-port 4x10G | 2 | 30–60 |
| 2 | Switch 48-port PoE+, 4x10G | 22 | 30–60 |
| 3 | Switch 24-port PoE+ 4x10G | 9 | 30–60 |
| 4 | Stack KIT | 24 | 30–60 |
| 5 | 50CM Stacking Cable | 21 | 30–60 |
| 6 | 100CM Stacking Cable | 3 | 30–60 |
| 7 | 10G SFP Module, Enterprise-Class | 32 | 30–60 |
| 8 | 1GSFP Module, Enterprise-Class | 18 | 30–60 |
| 9 | Power supply unit 600W (for hot spare) | 4 | 30–60 |
| 10 | Manufacturer technical support (Months) | 36 |  |