Technical Terms of Reference for a Wheel Loader for the "Bektakari" Underground Mine

CONTENTS

SECTION 1. GENERAL INFORMATION

Subsection 1.1 Name / Designation

Subsection 1.2 Information on Novelty

SECTION 2. SCOPE OF APPLICATION

SECTION 3. OPERATING CONDITIONS

SECTION 4. TECHNICAL REQUIREMENTS

Subsection 4.1. Key Technical, Economic, and Operational Performance Indicators

Subsection 4.2. Reliability Requirements

Subsection 4.3. Documentation Requirements

SECTION 5. REQUIREMENTS FOR THE SCOPE AND/OR PERIOD OF WARRANTIES

SECTION 6. MAINTAINABILITY REQUIREMENTS

SECTION 7. ENVIRONMENTAL REQUIREMENTS

SECTION 8. SAFETY REQUIREMENT

SECTION 1. GENERAL INFORMATION

1.1 Name / Designation

Universal self-propelled special equipment, a type of bucket loader designed for scooping, loading, and transporting various materials, as well as for executing quarrying and earthmoving operations.

1.2 Information on Novelty

The Supplier shall guarantee that the equipment supplied is new, previously unused, and a serial model incorporating all the latest modifications that has not been discontinued by the manufacturer at the time of delivery.

SECTION 2. SCOPE OF APPLICATION

For the transportation and unloading of materials.

SECTION 3. OPERATING CONDITIONS

For all macroclimatic regions on land, excluding climatic regions with an Antarctic cold climate.

| SECTION 4. TECHNICAL REQUIREMENTS |  |  |  |
| --- | --- | --- | --- |
| 4.1 Main Parameters and Dimensions |  |  |  |
| Length: |  |  | 7000 mm |
| Width (over bucket): |  |  | 2550 mm |
| Width (over wheels): |  |  | 2540 mm |
| Dump clearance / Discharge height (at 45° angle): |  |  | 2857 mm |
| Height (to top of cab): |  |  | 3340 mm |
| Bucket capacity: |  |  | 2 m³ |
| Payload capacity / Rated load: |  |  | 3500 kg |
| Engine |  |  |  |
| Engine type: |  |  | Liquid-cooled diesel |
| Maximum gross power: |  |  | 140–160 hp |
| Air intake system: |  |  | Multi-stage (1. Cyclone-type pre-cleaner; 2. Primary filter) |
| Exhaust system: |  |  | Catalytic converter, muffler |
| Starter voltage: |  |  | 24 V |
| Transmission |  |  |  |
| Transmission type: |  |  | Continuously variable hydrostatic transmission with intelligent control |
| Transmission modes: |  |  | 4 speed ranges with a smooth (infinitely variable) adjustment function |
| Axles |  |  |  |
| Front axle: | Fixed; locking differential |  |  |
| Rear axle: | Oscillating $\pm11$ degrees; open differential (standard); limited-slip differential (optional in certain regions) |  |  |
| Tires |  |  |  |
| Front wheels: | Tire size and type: 20-R25 L3 |  |  |
| Rear wheels: | Tire size and type: 20-R25 L3 |  |  |
| Braking System |  |  |  |
| Design / Configuration: | Spring-Applied Hydraulic-Released (SAHR) multi-disc brake |  |  |
| Service brake: | Enclosed hydraulic dual-circuit braking system; wet multi-disc brakes on both axles |  |  |
| Safety / Parking brake: | Fail-safe (reverse action), spring-applied, hydraulically released |  |  |
| Performance Characteristics |  |  |  |
| Forward and reverse: 1st gear: | From 1 to 13 km/h |  |  |
| Forward and reverse: 2nd gear: | 13 km/h |  |  |
| Forward and reverse: 3rd gear: | 27 km/h |  |  |
| Forward and reverse: 4th gear: | 40 km/h |  |  |
| Steering System |  |  |  |
| Steering type: Hydraulic articulated steering |  |  |  |
| Articulation angle (each direction): | 40° |  |  |
| Boom / Lift Arm |  |  |  |
| Attachment bracket / Coupler: | Hydraulic quick-attach coupler |  |  |
| Maximum lift height (to bucket hinge pin): | 3908 mm |  |  |
| Boom type: | Optimized Z-bar kinematics with parallel lift functionality |  |  |
| Attachments |  |  |  |
| Bucket: | 2 m³, payload capacity: 3,500 kg |  |  |
| Pallet forks: | Length: 1,200 mm; payload capacity: 3,500 kg |  |  |
| Electrical System |  |  |  |
| Operating voltage: | 24 В |  |  |
| Battery: | 2х12В |  |  |
| Protection rating: | 65 IP |  |  |
| Main and rear headlights: | LED |  |  |
| Auxiliary lights and indicator lamps: | LED |  |  |
| Reverse alarm and strobe light: | Audio and visual reverse signaling systems |  |  |
| Electrical wiring: | Wiring harnesses enclosed in fire-retardant and waterproof conduits (sheathing) |  |  |
| Operator Cabin |  |  |  |
| Cabin safety: |  | Falling Object Protective Structures (FOPS) and Roll-Over Protective Structures (ROPS) |  |
| External cabin glass protection against falling objects: |  | [Included / Required] |  |
| Operator seat: |  | Ergonomic air-suspension seat with a seatbelt |  |
| Visibility: |  | Equipped with a standard rearview camera |  |
| Other Equipment |  |  |  |
| Automatic fire suppression system: |  | [Included / Required] |  |
| Fire extinguisher: |  | Portable fire extinguisher, 2 x 8 kg |  |
| Information plates / Decals: |  | Warning plates and labels in Russian and English |  |
| Signal lights: |  | Flashing beacon / Strobe light |  |
| Spare wheel (with rim): |  | 2 pcs |  |
| Emergency braking system: |  | Hydraulic brake release (tow release) |  |
| Comfort: |  | Equipped with standard air conditioning, a heating system, a rear window defogger, and an intake air filtration system |  |
| 4.1 Technical, Economic, and Operational Indicators |  |  |  |
| Operating mode: On-demand / As required |  |  |  |
| 4.2 Reliability Requirements |  |  |  |
| Standard service life: Not less than 4 years |  |  |  |
| 4.3 Documentation Requirements |  |  |  |
| Operation, maintenance, repair, and diagnostic manuals in Russian, provided in both electronic and hard copy formats. Operation and maintenance instructions for the internal combustion engine, provided in both electronic and hard copy formats. Loader passport (technical data sheet / logbook). |  |  |  |
| - REQUIREMENTS FOR THE SCOPE AND PERIOD OF WARRANTY |  |  |  |
| Warranty period: Not less than 12 months, with no limitation on operating hours (under no hour limit). |  |  |  |
| - MAINTAINABILITY REQUIREMENTS |  |  |  |
| Loader components and their connections must withstand the forces and stresses to which they are subjected during active operation. The durability of the materials used must correspond to the intended operating conditions, taking into account hazards associated with fatigue, aging, corrosion, and wear phenomena. The machine's operation (instruction) manual must specify the type and frequency of inspection and maintenance required to ensure safety. Parts, assemblies, and components subject to wear must be specified, along with the criteria for their assessment and replacement. |  |  |  |
| - ENVIRONMENTAL REQUIREMENTS |  |  |  |
| Exhaust emissions must comply with the Euro 3 standard. |  |  |  |
| - SAFETY REQUIREMENTS |  |  |  |
| The wheel loader must be delivered in strict accordance with the "Safety Rules for Loading and Unloading Operations." |  |  |  |