| AGREED: Project Manager ___________ A. Patrakeev "___" __________ 2026 | APPROVED: Managing Director for Production Projects RMG Group JSC ___________ A. Nemokaev "___" __________ 2026 |
| --- | --- |

Technical Specification for the Procurement of Equipment

Flotation Concentrate Drying Furnace

Tbilisi 2026

- Purpose

# Dewatering of flotation concentrate by drying.

- Vendor Compliance Requirements

The equipment supplier shall be the manufacturer or an authorized representative and shall have experience in installation and maintenance, including at mining and processing plants.

- Technical Data

# 3.1 Main Requirements

| № | Technical Parameters | Main Technical Requirements |
| --- | --- | --- |
| Application Environment |  |  |
| 1 | Climatic category | Temperate |
| 2 | Operating air temperature, ℃ | No more than +40 |
| 3 | Duration of continuous operation per day, hours | 24 |
| 4 | Equipment utilization rate | 0,9 |
| Technical Characteristics |  |  |
| 5 | Dimensions (diameter × length), no more than, m | Ø2.4 × L10 (deviations are permitted by agreement with the customer as it limits the development area) |
| 6 | Quantity, pcs. | 1 |
| 7 | Cylinder installation angle, % | To be proposed at the stage of submitting the technical and commercial proposal |
| 8 | Cylinder rotation speed, rpm | To be proposed at the stage of submitting the technical and commercial proposal |
| 9 | Drying machine capacity, t/h | 20 |
| 10 | Maximum incoming air temperature, ℃ | To be proposed at the stage of submitting the technical and commercial proposal |
| 11 | Calculated temperature in direct-flow drying, ℃ | To be proposed at the stage of submitting the technical and commercial proposal |
| 12 | Gas and material flow regime | To be proposed at the stage of submitting the technical and commercial proposal |
| 13 | Process Automation (Automated Process Control System / APCS) | To be proposed at the stage of submitting the technical and commercial proposal |
| 14 | Drive type | Driven by a single electric motor |
| Electric Motor |  |  |
| 15 | Ingress Protection rating, IP | 55 |
| 16 | Voltage, V | 380 |
| 17 | Nominal network frequency, Hz | 50 |
| 18 | Explosion-proof design | No |
| 19 | Availability of frequency regulation | Yes |
| Feed Characteristics |  |  |
| 20 | Feed material | Copper-gold-bearing flotation concentrate (Cu = 6–15%, Au = 20–50 g/t) |
| 21 | Grain-size distribution | Content of the fine fraction -44 µm in the flotation concentrate is 74–75%. |
| 22 | Sulfur content, % | 32 % |
| 23 | Moisture content of the feed to be dried, % | No more than 18 |
| 24 | Target moisture content after drying, % | No more than 10 |
| Control Cabinet |  |  |
| 25 | Supply voltage, V | 220 V AC |
| 26 | Cabinet IP rating | IP65 or higher |
| The equipment must be provided with a full set of automation tools for the complete operation of the main equipment, including the implementation of all process and technical interlocks and protections. The control system must be based on a controller and an HMI (control panel) by Allen Bradley, Siemens, or an equivalent with comparable specifications (subject to customer approval). The cabinet must be designed to allow for a future 10% expansion of the number of I/O (input/output) channels and the corresponding installation of additional equipment (terminal connectors, galvanic isolators, controller chassis/rack or mounting rail, and power supplies). The control cabinet must be equipped with a touchscreen interface with a minimum diagonal of 12 inches to enable equipment control, logging of alarm and information messages, as well as trend monitoring. All field equipment must be hardwired to the control cabinet to transmit standard electrical signals (4-20 mA, +24 V DC, dry contact). The use of any digital protocols for transmitting signals from field equipment to the PLC cabinet is strictly prohibited. |  |  |
| Installation Details |  |  |
| 27 | Location | Indoors |
| 28 | Fuel Type | Electricity / Gas |
| 29 | Requirements for the Equipment Package | Including dust and gas cleaning system |

  - Main Technical and Process Requirements for Material and Technical Resources (MTR)

    - The design and material characteristics of the components and systems shall comply with the climatic conditions and allow for the rapid replacement of worn parts at the installation site.

    - The main assemblies and units shall operate using all-season hydraulic and transmission lubricants and oils.

    - The guaranteed service life of the electric motor and gearbox shall be no less than 10,000 hours.

    - Provision of guaranteed operating lifetimes (MTBF) for the main assemblies and components of the drying furnace.

- Requirements for Electrical Equipment

Cable and wire products must be flexible (with stranded conductors) featuring flame-retardant insulation that retains its plasticity within a temperature range of +45 to -15 ℃.

- Overall Dimensions

To be determined by the supplier based on the selected internal overall dimensions.

- Warranty Period Conditions

The warranty period shall be 12 months from the date of completion of acceptance testing or 24 months from the date of delivery to the customer’s warehouse. In the event of equipment downtime caused by the supplier, the warranty period shall be extended by the duration of such downtime.

- Scope of Supply

| No. | Name | Quantity, units |
| --- | --- | --- |
| 1 | Drying furnace | 1 |
| 2 | Spare parts (ZIP) kit for maintenance and repairs during a 12-month operating period | 1 set |

- List of Required Documentation

The Supplier shall provide the Customer with the following technical documentation:

| No. | Documentation | Submission Period** |
| --- | --- | --- |
| 1 | Dimensional drawing | 10 days after contract signing |
| 2 | Operating and maintenance instructions | Delivered with the equipment |
| 3 | Spare parts catalog (electronic version) | Delivered with the equipment |
| 4 | Drawings for wearing parts | Delivered with the equipment |
| 5 | Technical passport of the equipment | 10 days after contract signing |
| 6 | Electrical diagrams | Delivered with the equipment |
| 7 | Certificates of conformity and origin | Delivered with the equipment |

** Drawings must be provided in electronic format as both .pdf and .dwg files.

Documentation shall be provided in English or Russian.

- Packaging Requirements

Not regulated.

- Additional Conditions

10.1 Indicate the scope of supply.

10.2 Provide a preliminary power consumption calculation.

- Involvement of Subcontractors

Permitted subject to the Customer’s approval.

- Delivery Terms

The Goods shall be delivered on FOB – Country of Origin terms (the specific port of shipment to be agreed upon with the Customer).

| Prepared by: |  |  |
| --- | --- | --- |
| Equipment Procurement Engineer |  | I. Sobolev |
| Approved by: |  |  |
| Automation / ICSS Engineer |  | R. Suiunov |
| Power Engineer |  | V. Kostylev |