APPROVED BY:

Production Director, RMG "Auramine"

___________________ Abashmadze V.V.

August 12, 2026

TECHNICAL SPECIFICATIONS

For the supply of truck scales for weighing heavy-duty vehicles up to 100 tons for the RMG "Auramine" production site at the Bektakari Mine

TECHNICAL SPECIFICATIONS

For the supply of truck scales for weighing heavy-duty vehicles up to 100 tons

| № | Section | Description of Specifications and Product Requirements |
| --- | --- | --- |
| 1 | Product Name | Electronic truck scale (above-ground, foundation-based), 18 m, capacity up to 100 tons, for weighing cement trucks. |
| 2 | Compliance with Quality and Safety Requirements | Product compliance with technical regulations, safety standards, and quality parameters must be confirmed by a certificate of conformity. The truck scale must meet regulatory requirements and be registered in the State Register of Measuring Instruments. |
| 3 | Manufacturing | The truck scale must be manufactured for the existing concrete foundation on the Customer's premises in accordance with the dimensions provided in the drawing attached to these Technical Specifications. |
| 4 | Completeness | - Set of modules for scale platform assembly. - Complete set of hardware/parts for scale installation. - Set of load cells, 8 units, rated for 40 tons capacity each. - Communication cable. - Junction box and cable wiring. - Installation diagram. - Complete set of documentation in Russian, including the truck scale passport/data sheet with a section for verification marks. |
| 5 | Product Warranty | The warranty period for the product shall be no less than 5 years from the date of commissioning of the scale. |
| 6 | Mandatory Documentation Included with Delivery | Certificate of Conformity, technical documentation in hard copy and electronic format in Russian/Georgian languages: - Operating Manual (indicating technical characteristics) in Russian/Georgian languages; - Technical description, including wiring diagrams; - Spare parts catalogue; - Truck scale passport/data sheet. |
| 7 | Commissioning Procedure | Commissioning of the truck scale shall be performed on the Buyer's premises by the Supplier's specialists. |
| 8 | Supplier Requirements | Must be a manufacturer of truck scales or an official authorized dealer of a truck scale manufacturer. Must have its own service centre and certified specialists qualified/authorized to perform maintenance on truck scales. Availability of a spare parts inventory in stock. Capacity and authorization to conduct training for the Buyer’s staff on the Buyer's premises using certified specialists. |
| 9 | Supplier Selection Criteria | Supplier must possess an in-house service centre and certified specialists authorized to maintain the supplied goods throughout the entire operating period. Demonstrated operational and supply experience for the offered goods to other enterprises. |

Prepared by:

Power Engineer	Bektakari Mine	RMG		 "Auramine" 		Pelisov I.

/Signed/