AGREED: 	APPROVED:

Production Director	Chief Engineer

of Bektakari Underground Mine	of Bektakari Underground Mine

RMG Auramine	RMG Auramine

___________ Abashmadze V.V.	___________ Dzhamanov T.

«____» ___________    2026.	«____» ___________    2026.

Technical Terms of Reference

Subject of Tender/Procurement:

Certified industrial equipment for safe inflation of large OTR (Off-The-Road) tires for mining equipment of size 29.5R25.

Purpose of Equipment:

Ensuring complete safety of personnel during initial inflation and bead seating of tubeless OTR tires on multi-piece rims with lock rings. Eliminating injury risks in case of possible tire burst or ejection of rim locking elements.

Parameters of Serviced Wheels:

- Tire size: from 12.00R20 to 29.5R25

- Outer tire diameter: up to 1,900 mm

- Section width: up to 770 mm

- Assembled wheel weight: up to 850 kg

- Operating inflation pressure: up to 6.5–9.0 bar

Mandatory Technical Requirements:

- Certification: Mandatory presence of European CE certificate of conformity, DGUV or OSHA standards. Equipment with a low protection class is not allowed for the tender.

- Design: Reinforced steel cage (Martins Industries MIC-MINE type or equivalent) or certified hydraulic lock ring restrainer. Internal space must have clearance for 29.5R25 wheel dimensions.

- Protective Coating: Hot-dip galvanizing or heavy-duty wear-resistant powder coating.

- Ergonomics: Presence of ramps or a cradle for rolling in the wheel using a loader/OTR tire handler. Design must provide rigid anchoring to the concrete floor foundation.

Automation and Safety Requirements:

- Remote Control: Presence of a remote electronic programmable inflation station (hose/cable length not less than 5 meters from the potential hazard zone).

- Interlock: Mechanical or pneumatic system preventing air supply when cage doors are open.

Delivery Terms:

- Accompanying Documentation: Product passport, assembly and operation manual, copy of CE certificate of conformity.

Prepared by: Chief Mechanic						 Melnikov D.A.