JSC "RMG GROUP" • TECHNICAL SPECIFICATION

###### Hydrocyclone Cluster Unit Procurement

DOCUMENT ID

P01.C.26.10.202
LOCATION

Tbilisi, Georgia

ISSUE DATE

2026

1. PURPOSE & APPLICATION

This Technical Speciﬁcation deﬁnes the mandatory engineering and performance requirements for the procurement of a complete Hydrocyclone Cluster Unit (Hydrocyclone Plant). The equipment is intended for continuous dewatering of mineral processing tailings at the processing facility.

2. SUPPLIER QUALIFICATION REQUIREMENTS

Mandatory Supplier Criteria

The Supplier must be an Original Equipment Manufacturer (OEM) or an oﬃcially authorized direct representative with certiﬁed credentials.

Proven successful experience in engineering, installation, commissioning, and maintenance of cyclone clusters at mining and mineral processing plants.

3. TECHNICAL SPECIFICATIONS & OPERATING PARAMETERS

3.1 General Operational & Process Parameters

| # | Parameter / Parameter Description | Technical Requirement / Value |
| --- | --- | --- |
| Initial Feed Data |  |  |
| 1 | Plant Annual Capacity | 6.0 Mt / year |
| 2 | Tailings Output (Mass Flow Rate) | 762 t/h |
| Operating Environment & Conditions |  |  |
| 3 | Climatic Execution | Temperate climate region |
| 4 | Operating Ambient Temperature Range | -15 °C to +40 °C |
| 5 | Operating Schedule / Duty Cycle | 24/7/365 continuous (12h plant shutdown once/month) |
| 6 | Equipment Utilization Factor (EUF) | 0.95 minimum |
| Slurry Medium Characteristics |  |  |
| 7 | Slurry Abrasion Class | Medium abrasiveness |
| 8 | Nominal Slurry Volumetric Flow | 580 m³/h |
| 9 | Solid Phase Speciﬁc Gravity / Density | 2.75 t/m³ |
| 10 | Solid Phase Concentration | 20 % by weight |
| 11 | Material Type | Discarded tailings from copper ore processing |
| 12 | Maximum Solid Particle Size (Top Size) | 0.4 mm |
| 13 | Liquid type | Water |
| 14 | Type of pumped medium | Slurry (water + ground ore) |

| 1 | Hydrocyclone diameter, mm | 250 |
| --- | --- | --- |
| 2 | Location | Outdoors |
| 3 | Orientation | Vertical |
| 4 | Number of batteries | 2 |
| 5 | Number of hydrocyclones in operation | 44 |
| 6 | Standby hydrocyclones | 4 |
| 7 | Working pressure, kPa | 160 |

Hydrocyclone battery parameters:

3.2 Process Mass Balance Table

|  | Unit | Feed (Input) | Underﬂow (cyclone sands) | Overﬂow |
| --- | --- | --- | --- | --- |
| Solids Density | t/m³ | 2.75 | 2.75 | 2.75 |
| Solids Mass Flow Rate | t/h | 762.0 | 495.0 | 267.0 |
| Solids Volumetric Flow Rate | m³/h | 277.0 | 180.0 | 97.0 |
| Liquid Phase Density | t/m³ | 1.00 | 1.00 | 1.00 |
| Liquid Mass Flow Rate | t/h | 3,048.0 | 267.0 | 2,781.0 |
| Liquid Volumetric Flow Rate | m³/h | 3,048.0 | 267.0 | 2,781.0 |
| Slurry Bulk Density | t/m³ | 1.15 | 1.71 | 1.06 |
| Total Slurry Mass Flow Rate | t/h | 1,070.3 | 205.5 | 864.8 |
| Total Slurry Volumetric Flow Rate | m³/h | 3,325.0 | 447.0 | 2,887.0 |
| Solids Weight Concentration | % | 20.0 % | 65.0 % | 8.8 % |

3.3 Tailings particle size distribution (Sieve Analysis)

Ore grade: 11C, My - Mushevani, Ca - Sakrdisi

| Fraction Range (µm) | # 11С:Са:Му | # 11С:Са:Му | # 11С:Му | # 11С:Са:Му | # 11С:Са:Му | # 11С:Са:Му | # 11С:Са:Му | Average, % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # 01:03:01 | # 01:03:01 | # 02:01 | # 03:02:01 | # 03:02:01 | # 03:02:01 | # 03:02:01 |  |
| Date | 27.01.26 | 28.01.26 | 30.01.26 | 30.01.26 | 05.02.26 | 06.02.26 | 07.02.26 |  |
| 500 – 200 | 0.22 | 0.15 | 1.13 | 1.28 | 3.72 | 3.60 | 3.94 | 2.01 % |
| 200 – 140 | 0.32 | 0.80 | 0.28 | 2.63 | 1.47 | 3.06 | 3.82 | 1.77 % |
| 140 – 71 | 0.37 | 4.22 | 3.41 | 4.42 | 8.35 | 14.84 | 15.29 | 7.27 % |
| 71 – 32 | 42.16 | 39.79 | 37.36 | 41.35 | 33.63 | 30.12 | 29.79 | 36.31 % |
| 32 – 0 | 56.94 | 55.04 | 57.83 | 50.32 | 52.83 | 48.39 | 47.16 | 52.64 % |
| Total | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 | 100.00 % |
| Cumulative Passing -32 µm | 56.94 | 55.04 | 57.83 | 50.32 | 52.83 | 48.39 | 47.16 | 52.64 % |
| Cumulative Passing -71 µm | 99.10 | 94.83 | 95.18 | 91.67 | 86.46 | 78.50 | 76.95 | 88.96 % |

4. CONTROL SYSTEM ARCHITECTURE & INSTRUMENTATION

The HCU shall be designed for the maximum tailings feed rate. To maintain the specified pressure, each hydrocyclone battery shall include a pressure transmitter and a pressure gauge. The minimum required number of automatic shut-off valves, to be controlled by the plant’s upper-level control system (DCS), shall be calculated. All other shut-off valves shall be manually operated.

4.1 Detailed Instrumentation Speciﬁcations

1. Pressure Transmitter

Gauge pressure transmitter: pressure type — gauge pressure; connection — flange/diaphragm; measuring range — 0–10 bar; power supply — 24 V DC; standardized output signal — passive 4–20 mA; display for configuration; ingress protection rating of at least IP65; diaphragm and flange material — 316L stainless steel; shut-off valve upstream of the transmitter to allow removal without shutting down the equipment.

2. Dial Pressure Gauge

Pressure gauge: measuring range 0–600 kPa, case diameter 160 mm, radial diaphragm-seal connection (1½”), flushable design, diaphragm material — 316L stainless steel, case material — 304 stainless steel or higher, accuracy class 1, liquid-filled case, and an isolating valve upstream of the gauge to allow removal without shutting down the equipment.

3. Automated Actuated Valves & Control Logic

Actuator for shut-off valves: pneumatic; maximum operating pressure — 8 bar. The actuator shall include a solenoid valve designed for use with the specific pneumatic actuator; the shut-off valve opening/closing time shall be assumed to be 10 seconds. The solenoid valve shall be connected to the pneumatic cylinder using appropriate PVC tubing and compression fittings. The compression fitting for connection to the plant’s instrument air supply shall be suitable for 10 mm diameter PVC tubing. The solenoid valve shall have two coils with a 24 V DC control voltage.

In addition, the automatic shut-off valve assembly shall include non-contact limit switches of the PNP type, control voltage 24 V DC. The wires from the solenoid valve coils and limit switches shall be routed and terminated in a junction box mounted on the shut-off valve. The junction box shall contain a sufficient number of DIN-rail terminal blocks for the required electrical connections. The junction box material shall be metal or plastic; the box cover shall be secured with screws. An additional cable gland shall be provided in the box for connection of the DCS control cable (cable diameter: 8–10 mm). The ingress protection rating of all components on the shut-off valve shall be at least IP65.

An automatic throughput control system shall be provided for the hydrocyclone unit (HCU). For this purpose, pneumatically actuated gate valves shall be installed at the inlet to some of the hydrocyclones. The tailings pumping unit feeding the thickening stage operates on a level-control loop for the receiving tank (existing control scheme). When the level decreases, a command is issued to reduce the throughput; when the level increases, the throughput is increased. Accordingly, the pressure at the inlet to the hydrocyclone battery will also change. The main condition for stable hydrocyclone operation is maintaining constant pressure. Therefore, a pressure control loop at the hydrocyclone inlet shall be provided: some hydrocyclones shall be equipped with pneumatically actuated inlet gate valves. Based on the pressure transmitter readings in the hydrocyclone battery distributor, the system shall monitor the pressure. If the pressure falls below the setpoint, the system shall shut down hydrocyclones step by step; if the pressure rises, it shall bring hydrocyclones into service step by step. The design throughput per 250 mm hydrocyclone is 70–100 m³/h. Therefore, the HCU throughput control step is 70–100 m³/h, which represents 2–3% of the nominal throughput to ensure smooth control of HCU operation

5. ELECTRIC GENERAL REQUIREMENTS

Electric Motors: IP55 protection rating, 380V supply, 50 Hz nominal frequency. Electrical cables must be ﬂexible multi-core copper with ﬂame-retardant insulation retaining elasticity from -15 °C to +45 °C. All major lubricated units must use standard all-weather hydraulic and transmission oils.

6. COMMERCIAL & DELIVERY SCOPE

| Item | Description | Qty |
| --- | --- | --- |
| 1 | Hydrocyclone Cluster Unit | 2 Clusters |
| 2 | 24-Month Operating Spare Parts | 1 Set |

Scope of Supply

Warranty & Delivery Terms

Warranty: 12 months from commissioning or 24 months from warehouse delivery. Downtime extends warranty.

Incoterms: EXW or DP Kazreti, Georgia

Required Technical Documentation Schedule

| № | Document Description | Delivery Deadline* |
| --- | --- | --- |
| 1 | General Arrangement | Within 10 days of the date of contract signature |
| 2 | Operating and maintenance manuals | Supplied with the equipment |
| 3 | Spare Parts Catalog | Supplied with the equipment |
| 4 | Drawings for wear-prone components | Supplied with the equipment |
| 5 | Technical passport | Within 10 days of the date of contract signature |
| 6 | Electrical schematic diagrams | Supplied with the equipment |
| 7 | Certificates of conformity and origin | Supplied with the equipment |

* Note: All documentation must be delivered in dual language format (English and Russian).